Kathrein Inc., Scala Division Post Ofﬁce Box 4580 Medford, OR 97501 (USA) Phone: (541) 779-6500 Fax: (541) 779-3991Email: communications@kathrein.com Internet: www.kathrein-scala.com10907-C 936.3540* Mechanical design is based on environmental conditions as stipulated in TIA-222-G-2 (December 2009) and/or ETS 300 019-1-4 which include the static mechanical load imposed on an antenna by wind at maximum velocity. See the Engineering Section of the catalog for further details. General speciﬁcations : Frequency range 1710–2200 MHzImpedance 50 ohmsVSWR < 1.5:1Intermodulation (2x20w) IM3: <-150 dBcPolarization +45° and -45°Front-to-back ratio >30 dB (co-polar) >25 dB (total power)Maximum input power 300 watts per input (at 50°C)Electrical downtilt 0–10 degrees continuously adjustable (manual or optional remote control)Connector 4 x 7-16 DIN female Isolation >30 dBCross polar ratio Main direction 0° 25 dB (typical) Sector ±60° >10 dBWeight 32.2 lb (14.6 kg)Dimensions 51.9 x 12.7 x 2.8 inches (1319 x 323 x 71 mm)Wind load at 93 mph (150kph) Front/Side/Rear 147 lbf / 23 lbf / 158 lbf (650 N) / (100 N) / (700 N)Wind survival rating* 120 mph (200 kph)Shipping dimensions 62 x 14.2 x 5.1 inches (1574 x 360 x 130 mm)Mounting Fixed and tilt mount options are available for 2 to 4.6 inch (50 to 115 mm) OD masts. See reverse for order information. Speciﬁcations : 1710–1880 MHz 1850–1990 MHz 1920–2200 MHz Gain 17.6 dBi 17.8 dBi 18 dBi+45° and -45° polarization 64° (half-power) 64° (half-power) 62° (half-power) horizontal beamwidth+45° and -45° polarization 7° (half-power) 6.8° (half-power) 6.5° (half-power) vertical beamwidth Vertical Pattern – Sidelobe 0° 5° 10° T 0° 5° 10° T 0° 5° 10° T suppression for ﬁrst 20 18 16 dB 20 18 16 dB 16 18 16 dB sidelobe above main beam 742 236 hpbw: 65° Panel Antenna Vertical pattern ±45°- polarizationHorizontal pattern ±45°- polarization Kathrein panels are antenna products for wireless communications. All ADT panels feature:• Heavy-duty construction using the highest quality materials: UV resistant pulltruded ﬁberglass radomes, stainless steel fasteners and hardware, rigid aluminum reﬂectors, and low loss coaxial cable power divider harness, all contributing to long, trouble free service life.• Superior electrical speciﬁcations including low VSWR, wide bandwidth, ﬂat frequency response, and superb intermodulation performance.• Precision continuously adjustable electrical downtilt controls, manual control as standard equipment, with remote control available as an option.• Will accomodate future 3G / UMTS applications.

Kathrein Inc., Scala Division Post Ofﬁce Box 4580 Medford, OR 97501 (USA) Phone: (541) 779-6500 Fax: (541) 779-3991Email: communications@kathrein.com Internet: www.kathrein-scala.com10907-C 936.3540* Mechanical design is based on environmental conditions as stipulated in TIA-222-G-2 (December 2009) and/or ETS 300 019-1-4 which include the static mechanical load imposed on an antenna by wind at maximum velocity. See the Engineering Section of the catalog for further details. General speciﬁcations : Frequency range 1710–2200 MHzImpedance 50 ohmsVSWR < 1.5:1Intermodulation (2x20w) IM3: <-150 dBcPolarization +45° and -45°Front-to-back ratio >30 dB (co-polar) >25 dB (total power)Maximum input power 300 watts per input (at 50°C)Electrical downtilt 0–10 degrees continuously adjustable (manual or optional remote control)Connector 4 x 7-16 DIN female Isolation >30 dBCross polar ratio Main direction 0° 25 dB (typical) Sector ±60° >10 dBWeight 32.2 lb (14.6 kg)Dimensions 51.9 x 12.7 x 2.8 inches (1319 x 323 x 71 mm)Wind load at 93 mph (150kph) Front/Side/Rear 147 lbf / 23 lbf / 158 lbf (650 N) / (100 N) / (700 N)Wind survival rating* 120 mph (200 kph)Shipping dimensions 62 x 14.2 x 5.1 inches (1574 x 360 x 130 mm)Mounting Fixed and tilt mount options are available for 2 to 4.6 inch (50 to 115 mm) OD masts. See reverse for order information. Speciﬁcations : 1710–1880 MHz 1850–1990 MHz 1920–2200 MHz Gain 17.6 dBi 17.8 dBi 18 dBi+45° and -45° polarization 64° (half-power) 64° (half-power) 62° (half-power) horizontal beamwidth+45° and -45° polarization 7° (half-power) 6.8° (half-power) 6.5° (half-power) vertical beamwidth Vertical Pattern – Sidelobe 0° 5° 10° T 0° 5° 10° T 0° 5° 10° T suppression for ﬁrst 20 18 16 dB 20 18 16 dB 16 18 16 dB sidelobe above main beam 742 236 hpbw: 65° Panel Antenna Vertical pattern ±45°- polarizationHorizontal pattern ±45°- polarization Kathrein panels are antenna products for wireless communications. All ADT panels feature:• Heavy-duty construction using the highest quality materials: UV resistant pulltruded ﬁberglass radomes, stainless steel fasteners and hardware, rigid aluminum reﬂectors, and low loss coaxial cable power divider harness, all contributing to long, trouble free service life.• Superior electrical speciﬁcations including low VSWR, wide bandwidth, ﬂat frequency response, and superb intermodulation performance.• Precision continuously adjustable electrical downtilt controls, manual control as standard equipment, with remote control available as an option.• Will accomodate future 3G / UMTS applications.